ELECTRONICS DESIGN REPORT

M A R S

Team division :

- Control system

- Communication

- Power electronics

# CONTROL SYSTEM

- The aim of the team is to design and build controllers for the rover's robotic arm and mobility unit.

- The mobility controller and manipulator controller are the two main controllers we have to build.

## 1)	MOBILITY CONTROLLER

DESIGN PHASE :

- We started from

- a requirements phase, from the requirements we started building a controller which satisfied the requirements .

- In the design phase we mainly stick to MATLAB for designing and testing the control designs .

- In matlab we started , building the controllers for dc motor speed control to the whole mobility controller .

- Here is the mobility controller model , where the controller will get the feedback from the rover model which is created using multibody .

- In matlab , we created an environment using simulink multibody for testing the controller. ●	We imported a rover design from onshape to matlab multibody to test the controller.

- This environment used a multibody multiphysics library used to create frictional forces .

- After the environment is created , we can start testing the controller , to control the rover in the environment .

- Our controller design embedded a cross coupled controller for more precise control .

- We designed a controller , which can control two motors, left and right , so these controllers can be used to control all 6 motors of the rover .

- Here in the cross coupled controller pid_1&2 are responsible for controlling the speed of each motor.

- The pid_3 is responsible for maintaining the relative speed between left and right motors . it is also responsible for eliminating the deflection of the rover from the straight line of motion

- Below  is a transfer function of the motor , which gives the torque output from the input voltage, such that we can feed the torque as input to the environment model .

Where :

Kt      =      torque constant

J        =     moment of inertia

R       =      resistance of coil

Kb     =      back emf constant

- Here is the , final mechanical explorer view of the simulation of rover

You can find the simulation video in MARS DRIVE UNDER ELECTRONICS

- Finally we tested and verified the controller . so now we have to embed this design to the real hardware and software

- So , we planned to use a stm32f103c8 microcontroller for implementing each cross coupled controller , which will control two wheels in the rover . so , total 3 stm’s and a nano which is responsible for communicating with three stm’s and with the main processor and also provide imu data .

IMPLEMENTATION PHASE :

- In this phase , we are responsible for code the individual controllers so as to act like a controller that we designed .

- So , we splitted this phase into 3

  - Fetching & decoding the dc motor encoder data, for feedback of speed and position .

  - Coding the controller according to the design in matlab

  - Enabling the i2c bus communication for getting commands and data from the master .

ENCODERS  DATA :

- We are using a quadrature encoder for feedback , where we have to read two signals from the encoder .

- There will be a lag between two signals which is responsible for detecting the direction of the motor rotation .

- We are using a stm31 to read the four signals from two encoders .

- In stm , we are enabling external picchange intercepts , for reading the encoder data .

- Here is the arduino code for stm.

////////////////// READING ENCODER1 DATA USING THE SYSTICK TIMER(MILLIS) /////////////////////

void ENCODER1() { /// determining value of micros in the time of an interrupt, where micros won't update

current_state1  = MICROS(); pulse_time1     = current_state1 - previous_state1; previous_state1 = current_state1;

////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

if((GPIOB_BASE->IDR >> (uint32_t)13) & 0x1)  DIR1 = 1; // reading PB13 pin  for second

encoder signal for direction

else DIR1 = -1;

F1 = DIR1*(double(1000000/double(pulse_time1*N*G))); E1 = E1 + DIR1;

}

- The above code will execute , when a rising  edge is detected in the encoder signal .

- Inside the function , we were measuring the time in which interrupt occurred , and stored in current_state_1 ,

- Then , we determined the time period of the encoder pulse to calculate the frequency  .

- Every time an edge is detected , the variable E1 increments by 1 .

- We were using a custom made function MICROS() , because if we use micros inbuilt function inside interrupt, NVIC creates a problem , where the millis counter won't update such that we will get the wrong time period .

/////////////////// MICROS FUNCTION WHICH WORKS INSIDE INTERRUPTS //////////////////// uint32_t MICROS(void) {

uint8_t  int_flag;

int32_t  systick_counter,current_state;

systick_counter = SYSTICK_BASE->CNT;

// System timer(Systick) base

if(0b1 & SCB_BASE->ICSR >>26){ //Check if Systick interrupt pending flag is set

int_flag = 1;

systick_counter = SYSTICK_BASE->CNT;

//Re-read Systick Timer

}

else int_flag = 0;

// Systick interrupt pending flag is not set

current_state = (systick_uptime_millis * 1000) + (SYSTICK_RELOAD_VAL + 1 systick_counter) / CYCLES_PER_MICROSECOND;

if(int_flag) current_state += 1000;

return current_state;

}

For more reference , to understand the above code refer link :

STM32 for Arduino - Connecting an RC receiver should be easy, right?

CONTROLLER CODING IMPLEMENTATION :

- We needed to code the controller design , so we coded the controller in arduino ide for stm32f103 .

- We splitted the coding part into two parts :

1) Coding the cross coupled motor controller .

- This code should be implemented in a stm32.

- This controller code is responsible for controlling the two left & right motors by coding the desired controller according to matlab .

- We were using PD   as a base controller

2) Coding mobility master controller

- This master is responsible for running dead reckoning algorithms and fetching data from IMU and the main communication node .

- We will use an arduino nano for this master .

- This controller will fetch data from the main communication node via can protocol .

//-------------------------- CROSS COUPLED CONTROLLER ------------------------------//

// STEERING PID FOR FRONT MOTORS

ERROR_S      = (L_M_VEL - R_M_VEL) - (FEEDBACK_L - FEEDBACK_R) -

FEEDBACK_STEER;

//----------------------------------------------------------------------------------------------------------------

S_PID      = PID(ERROR_S, PREV_ERROR_S, &S_I,S_Kp, S_Ki,S_Kd, SAT_S_PID,

S_PID,TIMER1);	// PID FUNCTION USAGE

//----------------------------------------------------------------------------------------------------------------

TIMER1       = micros();

SAT_S_PID    = SATURATE(S_PID,-SAT_PID_VALUE,SAT_PID_VALUE);	// SATURATE THE PID

PREV_ERROR_S = ERROR_S;

// ********************************************************************

- The above code is pid_3 which is responsible for maintaining relative speed between the two motors

- This pid feedback also gets the imu data , to avoid the deflection . ●	However, the pid_3 controller is also a PI controller .

// PID CONTROLLER FOR LEFT FRONT MOTOR

ERROR_L      = L_M_VEL - FEEDBACK_L - (S_PID * 0.3);

//------------------------------------------------------------------------------------------------------------L_PID        =

PID(ERROR_L,PREV_ERROR_L,&L_I,L_Kp,L_Ki,L_Kd,SAT_L_PID,L_PID,TIMER2); // PID

FUNCTION USAGE

//-------------------------------------------------------------------------------------------------------------

TIMER2       = micros();

SAT_L_PID    = SATURATE(L_PID,-SAT_PID_VALUE,SAT_PID_VALUE);	// SATURATE THE PID

PREV_ERROR_L = ERROR_L;

// *******************************************************************

// PID CONTROLLER FOR FRONT RIGHT MOTOR

ERROR_R      = R_M_VEL - FEEDBACK_R + (S_PID * 0.3);

//------------------------------------------------------------------------------------------------------------R_PID        =

PID(ERROR_R,PREV_ERROR_R,&R_I,R_Kp,R_Ki,R_Kd,SAT_R_PID,R_PID,TIMER3); // PID

FUNCTION USAGE

//-------------------------------------------------------------------------------------------------------------

TIMER3       = micros();

SAT_R_PID    = SATURATE(R_PID,-SAT_PID_VALUE,SAT_PID_VALUE);	// SATURATE THE PID

PREV_ERROR_R = ERROR_R;

}

- The above code is a PI controller , which is responsible for maintaining a desired set speed .

- We used a custom PID( ) function for implementing the above code .

- This pid function also eliminates the integral windup problem , and saturation problem  .

- This controller gets the input data from the main communication node via I2C master arduino nano .

///////////////////////////////////////// PID FUNCTION /////////////////////////////////////

double PID(double ERROR_, double PREV_ERROR_, double *I,byte Kp,byte Ki,byte Kd,double SAT_PID,double PID,uint32_t TIMER){ // calculate the pid according to gains .

double P,D; double FREQ;

FREQ = micros() - TIMER;

P = ERROR_ * (double)Kp;	//    finding proportional *I = *I + ERROR_ * (Ki / FREQ);	// finding integral

*I = *I + 2*(SAT_PID - PID);	// eliminating the integral windup .

D = (PREV_ERROR_ - ERROR_) * (double)Kd * FREQ;	// finding derivative term

return (P + *I + D);

}

MAIN CONTROLLER CODE :

●	This main controller is responsible of doing the following tasks :

    - Receiving data from the main communication node thusing can protocol and sending it to stm’s

    - Dead reckoning

    - Collecting data from IMU

- Receiving data from main communication node

  - We will receive data from a MCP2515  can module ●	We will communicate with spi with the module .

- Dead reckoning

  - This code is responsible for determining the position of the rover with respect to a starting position .

  - It will use encoder data and imu data to determine its location

Below is the corresponding code for dead reckoning

/////////////// DETERMINING THE AMOUNT OF DISTANCE THE ROVER HAD MOVED /////////////// DELTA_D = (DELTA1 + DELTA2 + DELTA3 + DELTA4 + DELTA5 + DELTA6 )/(6*N*G); // DETERMINING THE AVERAGE ENCODER COUNT ...

DELTA_D = (DELTA_D + (RPM1 + RPM2 + RPM3 + RPM4 + RPM5 + RPM6)/(F*60*6))/2;	// APPROXIMATING ROVERS MOTION THROUGH VELOCITY DATA ...

DELTA_D = DELTA_D*(2*3.1415926*WHEEL_RADIUS);

// UNITS IN "mm"

///////////////// DETERMINING THE COORDINATES ///////////////////////////////////////

X1 = X1 + DELTA_D*sin(YAW*DEG_RAD)*cos(PITCH*DEG_RAD);	// units in mm Y1 = Y1 + DELTA_D*cos(YAW*DEG_RAD)*cos(PITCH*DEG_RAD);	// units in mm

Here X1 and Y1 are the final coordinates of the rover .

3) Collecting data from IMU

- We were using an adafruit IMU for getting the whole yaw , pitch and roll orientation data of the rover .

I2C COMMUNICATION :

- Here , we are using 3 stm’s for controlling 6 motors , so there is a master controller which is responsible for running dead reckoning algorithms and fetching data from IMU and the main communication node .

- So, we are using I2C communication for sending data from master to the stm’s .

- In this communication the stm’s are coded as I2C slaves .

- In this communication , we were sending encoder data to master and receive , speed commands for two motos .

THESE ARE FEW DOCS WE PREPARED WHILE DESIGNING PHASE

LINKS :

- MARS CONTROLL DESIGNS

- MARS ROVER TEAM

- Control system design

- Power modules and devices

- MARS  ROVER STEERING CONTROL DESIGN

- SENSOR FUSION

- Motors

2) MANIPULATOR CONTROLLER

- This  manipulator controller's main aim is to position the manipulator joints to its appropriate location .

- So, this controller has two modes :

  - Getting each joint angle from an external processor (ie.. getting the joint angle data from a rovers on board processor using  inverse kinematic algorithm ).

  - (Or)  determining its own joint angles from 3 DOF inverse kinematic  algorithms running inside the controller (This  feature can be used when the processor algorithm fails ).

### Design phase

- In this phase , our main aim is to design a joint controller in matlab for a manipulator .

- Such that we created a position controller in matlab

- Our main control aim is to develop a current based position controller .

- According to that we developed a controller which controls speed and position

#### Controller

In the above , the results of the both controllers will together command the joint .

- The third sum is , feedforward path which will provide the appropriate torque to each joint to eliminate the effect due to gravity .

Feed forward path :

These are the matlab code for determining the gravitational torque acting on three joints

MATLAB CODE :

MAIN_FUNCTION CODE :-

SUB_FUNCTION code :

This function takes inputs as angles of three joints , and  performs some tegromentric calculations to determine the effective center of mass of the whole three joints .

From that data , we can determine the torques , acting on each joint from manipulator parameters and the data from the main function .

So , these remaining functions are responsible for calculating the torques on each joint .

- CREATING THE SIMULATION ENVIRONMENT :

1. We created a manipulator , using simscape multibody to test our controller with each joint .

This is the overall view of the whole system .

We simulated using mechanics explorer in matlab ,

This is the final output of the simulation environment of the manipulator  which we test . In the above file each joint had his own controller , to control the manipulator .

IMPLEMENTATION PHASE :

- In this phase we have to create an architecture for the manipulator controller , with appropriate microcontrollers .

- So , our architecture consists of two stm's, one responsible for controlling the end effector and another responsible for controlling below 3 joints .

- And one arduino nano as main controller for both stm’s for transferring data from the main communication node .

Stm controller  code for end effector :

// ----------------------      - FINDING GRIPPER MOTOR TORQUE -------------------------------------

GRIPPER_TORQUE = GRIPPER_FEEDBACK_I*Kt;	// MOTOR TORQUE

//---------------------------------------------------------------------------------------------------------------

// %%%%%%%%% GRIPPER TORQUE CONTROLLER  %%%%%%%%%%%

GRIPPER_ERROR = GRIPPER_SET  - GRIPPER_TORQUE;	// FINDING ERROR

GRIPPER_PID = PID(GRIPPER_ERROR, GRIPPER_PREV_ERROR, &GRIPPER_I,

GRIPPER_Kp, GRIPPER_Ki, GRIPPER_Kd, GRIPPER_SAT_PID, GRIPPER_PID, G_T); //

CALCULATING PID

GRIPPER_PREV_ERROR = GRIPPER_ERROR;

GRIPPER_SAT_PID  = SATURATE(GRIPPER_PID,-SAT_PID_VALUE,SAT_PID_VALUE);

//%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

G_T = micros();

This controller code is responsible for maintaining a constant gripping force on the gripper when activated .

It will get current data from the current sensors for the motor current to determine the torque .

// ********************* YAW_ANGULAR_SPEED_PID *************************************

YAW_W_ERROR = YAW_SET_W  - YAW_FEEDBACK_W;

YAW_PID_W = PID( YAW_W_ERROR, YAW_W_PREV_ERROR, &YAW_W_I, YAW_W_Kp,

YAW_W_Ki, YAW_W_Kd,YAW_SAT_PID_W,YAW_PID_W,TIMER_YAW);

YAW_W_PREV_ERROR = YAW_W_ERROR;

YAW_SAT_PID_W = SATURATE(YAW_PID_W,-SAT_PID_VALUE,SAT_PID_VALUE);

// ******************************* YAW_POSITION_PID *************************************

YAW_T_ERROR = (YAW_SET_T  - YAW_FEEDBACK_T)*0.1 - YAW_FEEDBACK_I;

YAW_PID_T = PID( YAW_T_ERROR,YAW_T_PREV_ERROR, &YAW_T_I, YAW_T_Kp,

YAW_T_Ki, YAW_T_Kd,YAW_SAT_PID_T,YAW_PID_T,TIMER_YAW);

YAW_T_PREV_ERROR = YAW_T_ERROR;

YAW_SAT_PID_T = SATURATE(YAW_PID_T,-SAT_PID_VALUE,SAT_PID_VALUE);

TIMER_YAW = micros();

The above controller , had a position controller and a speed controller which is responsible for which appropriately control the joint angle . The position controller uses current based control .

// ********************************* JOINT_5_POSITION_PID *************************************

JOINT_5_T_ERROR = (JOINT_5_SET_T  - JOINT_5_FEEDBACK_T)*0.1 -

JOINT_5_FEEDBACK_I;

JOINT_5_PID = PID( JOINT_5_T_ERROR, JOINT_5_T_PREV_ERROR, &JOINT_5_T_I,

JOINT_5_T_Kp, JOINT_5_T_Ki, JOINT_5_T_Kd,JOINT_5_SAT_PID,JOINT_5_PID,TIMER_5);

JOINT_5_T_PREV_ERROR = JOINT_5_T_ERROR;

JOINT_5_SAT_PID = SATURATE(JOINT_5_PID,-SAT_PID_VALUE,SAT_PID_VALUE);

TIMER_5 = micros();

This joint 5 and 4 controller had only a current based position controller , because it will gate the commands from an inverse kinematic controller inside the main controller .

- When the rover is in autonomous mode , we can't give instructions to the manipulator from the base station , so the prosser has to give the data then in this mode each joint controller will implement only the current based position controller .

- When we are controlling the manipulator from the base station, then we only get the speed of each joint rather than position , at this time we will implement the speed controller along with the position controller for three joints .

- But for joints 5, 6 they always implement only with current based position controllers .

INVERSE KINEMATIC CODE :

- This inverse kinematic code is implemented in the main controller

////////////////////// FORWARD  KINEMATICS  /////////////////////////////

- = L1*sin(THETA4) + L2*sin(THETA5) + double(CAN_J_4)/F;

- = L1*cos(THETA4) + L2*sin(THETA5) + double(CAN_J_5)/F;

///////////////////// INVERSE KINEMATICS ///////////////////////////////

T2     = acos((X^2 + Y^2 - L1^2 - L2^2)/(2*L1*L2));

J4_SET = T2;

K1     = L1 + L2*cos(T2);

K2     = L2*sin(T2);

T1     = atan2(X/Y) - atan2(K1/K2);

J5_SET = T1;

We are only determining the two joint angles from the inverse kinematics.

In forward kinematics , we are updating the X and Y positions from user commands . Then we can apply inverse kinematics to determine the angles .

So the user has control over X , Y , YAW , END EFFECTOR ORIENTATION , JOINT_3 AND GRIPPER .

So , the user can move the manipulator in X and Y direction and he has to control the bottom joint to rotate the manipulator .

User has another three controls to control the end effector orientation and the gripper actions .